Mesh node site survey ARTD2144 Interactivity and Play, Year 2 Creative Computing, Winchester School of Art Thursday walk: October Books to Highfield, May 2026

What this sheet is A walking field guide for picking where four solar powered mesh network nodes could go between October Books and Highfield. We'll stop at four sites. At each one, you'll notice five things about the place. The questions are observational. The engineering vocabulary comes in afterwards as a name for what you noticed. A half-filled sheet with thoughtful answers is better than a full sheet with shallow ones. The walk is the tool. This is just a prompt for noticing. Words you'll meet today LoRa, Meshtastic, RSSI, Site Planner, antenna height, line of sight, clutter, node role. Don't worry about these now. We'll name them as we go.

| Surveyor |  | Date / time |  |
| --- | --- | --- | --- |
| Pair |  | Weather |  |

Before we leave October Books

We'll run the Meshtastic Site Planner together (site.meshtastic.org). It's a computer tool that draws a coloured map showing where the radio signal from October Books is predicted to reach: green for strong, yellow for marginal, red for weak. The map is a guess based on physics calculations. It uses terrain data but doesn't see trees or buildings, so it's a prediction to test against the walk, not a truth.

Three predictions to record before we leave

Look at the map together. Write down what it predicts, so we can compare later.

| Where does the map say we'll have strong signal? Which parts of Portswood Road show green? |  |
| --- | --- |
| Where does the map say signal will be marginal or weak? Which parts show yellow or red? |  |
| Does anything about the prediction look suspicious? Remember: the map can't see trees or buildings. |  |

The numbers we plug into the planner

These describe our actual setup at October Books. You don't need to remember them, just notice that each number is tied to a real, physical thing.

| Location | October Books, 50.9253 N, 1.3936 W |
| --- | --- |
| Antenna height | 3 metres, ideally just outside the shopfront (outside beats inside) |
| Frequency | 868 MHz (the European LoRa band) |
| Transmit power | 22 dBm (the maximum allowed in the UK) |
| Antenna gain | 2 dBi (the stock aerial that comes with the board) |
| Receiver sensitivity | -133 dBm (how weak a signal Meshtastic can still hear) |
| Obstruction height | 8 metres (typical Portswood terraces and trees) |

| Site 01 | Around October Books area Roughly where: street, landmark, anything that pins it down |
| --- | --- |

| 1. Feel the place Stand here for a moment. What's the first thing you notice? Is it busy or quiet? Bright or shady? Exposed or sheltered? Where is the sun coming from right now? engineers would call this: orientation, solar exposure, weather exposure |  |
| --- | --- |
| 2. Look up. Where could it go? If you had to fix a small box and a solar panel here, where would it actually go? Wall, lamp post, fence, tree branch, balcony, signage? About how high above the ground? Head height, first floor, second floor, higher? engineers would call this: antenna height and mounting affordance. Higher is better, outside is better than inside. |  |
| 3. Look toward the next site Look in the direction of the previous site (or, at site 1, the direction of site 2). Can you still see it from here? What's in the way? Buildings, trees, walls, open sky? Roughly how far away is it? Closer than the last gap, or further? How many minutes did the walk take? engineers would call this: line of sight, clutter, and link distance. Urban LoRa typically reaches 200 m to 1 km between obstacles. |  |
| 4. Who's here, who owns it Who passes by this spot? Residents, students, shoppers, no one? Whose wall or land is this? Who would notice if something happened to it, and who would we need to ask? engineers would call this: stewardship and permission. Not engineering, but it makes or breaks a deployment. |  |
| 5. Verdict Yes, no, or maybe, and one sentence on why. |  |

Sketch and photos Draw the spot. Mark where a node could go, where the sun comes from, what's in the way. Note photo filenames.

| Site 02 | Along Portswood Road Roughly where: street, landmark, anything that pins it down |
| --- | --- |

| 1. Feel the place Stand here for a moment. What's the first thing you notice? Is it busy or quiet? Bright or shady? Exposed or sheltered? Where is the sun coming from right now? engineers would call this: orientation, solar exposure, weather exposure |  |
| --- | --- |
| 2. Look up. Where could it go? If you had to fix a small box and a solar panel here, where would it actually go? Wall, lamp post, fence, tree branch, balcony, signage? About how high above the ground? Head height, first floor, second floor, higher? engineers would call this: antenna height and mounting affordance. Higher is better, outside is better than inside. |  |
| 3. Look toward the next site Look in the direction of the previous site (or, at site 1, the direction of site 2). Can you still see it from here? What's in the way? Buildings, trees, walls, open sky? Roughly how far away is it? Closer than the last gap, or further? How many minutes did the walk take? engineers would call this: line of sight, clutter, and link distance. Urban LoRa typically reaches 200 m to 1 km between obstacles. |  |
| 4. Who's here, who owns it Who passes by this spot? Residents, students, shoppers, no one? Whose wall or land is this? Who would notice if something happened to it, and who would we need to ask? engineers would call this: stewardship and permission. Not engineering, but it makes or breaks a deployment. |  |
| 5. Verdict Yes, no, or maybe, and one sentence on why. |  |

Sketch and photos Draw the spot. Mark where a node could go, where the sun comes from, what's in the way. Note photo filenames.

| Site 03 | Approach to Highfield Roughly where: street, landmark, anything that pins it down |
| --- | --- |

| 1. Feel the place Stand here for a moment. What's the first thing you notice? Is it busy or quiet? Bright or shady? Exposed or sheltered? Where is the sun coming from right now? engineers would call this: orientation, solar exposure, weather exposure |  |
| --- | --- |
| 2. Look up. Where could it go? If you had to fix a small box and a solar panel here, where would it actually go? Wall, lamp post, fence, tree branch, balcony, signage? About how high above the ground? Head height, first floor, second floor, higher? engineers would call this: antenna height and mounting affordance. Higher is better, outside is better than inside. |  |
| 3. Look toward the next site Look in the direction of the previous site (or, at site 1, the direction of site 2). Can you still see it from here? What's in the way? Buildings, trees, walls, open sky? Roughly how far away is it? Closer than the last gap, or further? How many minutes did the walk take? engineers would call this: line of sight, clutter, and link distance. Urban LoRa typically reaches 200 m to 1 km between obstacles. |  |
| 4. Who's here, who owns it Who passes by this spot? Residents, students, shoppers, no one? Whose wall or land is this? Who would notice if something happened to it, and who would we need to ask? engineers would call this: stewardship and permission. Not engineering, but it makes or breaks a deployment. |  |
| 5. Verdict Yes, no, or maybe, and one sentence on why. |  |

Sketch and photos Draw the spot. Mark where a node could go, where the sun comes from, what's in the way. Note photo filenames.

| Site 04 | Highfield campus edge Roughly where: street, landmark, anything that pins it down |
| --- | --- |

| 1. Feel the place Stand here for a moment. What's the first thing you notice? Is it busy or quiet? Bright or shady? Exposed or sheltered? Where is the sun coming from right now? engineers would call this: orientation, solar exposure, weather exposure |  |
| --- | --- |
| 2. Look up. Where could it go? If you had to fix a small box and a solar panel here, where would it actually go? Wall, lamp post, fence, tree branch, balcony, signage? About how high above the ground? Head height, first floor, second floor, higher? engineers would call this: antenna height and mounting affordance. Higher is better, outside is better than inside. |  |
| 3. Look toward the next site Look in the direction of the previous site (or, at site 1, the direction of site 2). Can you still see it from here? What's in the way? Buildings, trees, walls, open sky? Roughly how far away is it? Closer than the last gap, or further? How many minutes did the walk take? engineers would call this: line of sight, clutter, and link distance. Urban LoRa typically reaches 200 m to 1 km between obstacles. |  |
| 4. Who's here, who owns it Who passes by this spot? Residents, students, shoppers, no one? Whose wall or land is this? Who would notice if something happened to it, and who would we need to ask? engineers would call this: stewardship and permission. Not engineering, but it makes or breaks a deployment. |  |
| 5. Verdict Yes, no, or maybe, and one sentence on why. |  |

Sketch and photos Draw the spot. Mark where a node could go, where the sun comes from, what's in the way. Note photo filenames.

After the walk

Our four chosen spots, October Books to Highfield Site number, what's there, one line on why it works.

What role would each node play Now that you've seen all four sites, which is the best vantage point?

In Meshtastic, every node has a role. CLIENT is the polite default that helps relay when needed. ROUTER aggressively rebroadcasts everything and should only sit at a node with a genuinely good vantage point. The community guidance is: most nodes should be CLIENT. Only one of your four sites should be ROUTER, if any. Site 01 (role and one sentence why): Site 02 (role and one sentence why): Site 03 (role and one sentence why): Site 04 (role and one sentence why):

Where the map and the walk disagreed Did the Site Planner map predict coverage the walk made you doubt? Or the other way? A few sentences.

What we'd still need to check before installing anything Permission, access, signal verification, anything else.

A note for the teacher

This sheet is designed to do the engineering vocabulary in a particular order: experience first, name second.

Recommended sequence

At October Books, before the walk: Run the Site Planner together as a class. Fill in the three predictions on the planner page. Set up the reference node. Five minutes on the four key words (LoRa, Meshtastic, RSSI, Site Planner), just enough that students recognise them when they appear.

At site 1: Let students fill in the page using only the observational prompts. Don't unpack the green footers yet. Let them notice without performing expertise.

Between site 1 and site 2: Gather. Spend five minutes naming what they just did. "What you just noticed about what's in the way of the next site, engineers call clutter and line of sight. Higher and more open is better. Trees and brick walls absorb LoRa signal especially at 868 MHz." Now the vocabulary lands on something they already noticed.

Sites 2, 3, 4: Students fill the sheet using both layers, observational and technical. They'll start using the vocabulary themselves.

At Highfield, after the walk: Fill in the decision page together. The role question comes last because it can only be answered once all four sites have been seen against each other.

Things to bring

A printed route map with the four candidate sites marked. A tape measure (useful at site 2 or 3 if students want to actually measure something). Phones with the Meshtastic app installed for RSSI readings. The reference node, fully assembled, mounted just outside October Books if possible. Spare survey sheets in case anyone soaks theirs.

On precision

Year 2 art students don't carry tape measures and theodolites. The prompts ask for embodied estimation: head height, first floor, second floor. Closer than the last gap, or further. How many minutes the walk took. These are answerable by anyone standing there with a body and a watch, and they're good enough for a speculative survey. A node deployment that actually happens next year would need more precision, but that's a problem for the team that installs them.